COMPARATIVE ANALYSIS OF MEMORY TECHNOLOGIES AND HIERARCHICAL MEMORY SYSTEMS

Name: SHIVATHMIKA V

Register No.: 192521028
Subject: Computer Architecture
Date: 20-08-2026

1. COMPARISON OF L1, L2 AND L3 CACHE LEVELS

1.1 Introduction

Cache memory is a small and high-speed memory located close to the processor. It stores frequently accessed instructions and data so that the CPU does not have to repeatedly access slower main memory. Modern processors generally use multiple cache levels, commonly called L1, L2 and L3.

The three levels form a hierarchy. L1 is the smallest and fastest cache, L2 is larger but slower, and L3 is much larger but has higher access latency. This hierarchy attempts to provide a balance between speed, capacity and cost.

1.2 Comparison

| Parameter | L1 Cache | L2 Cache | L3 Cache |
| --- | --- | --- | --- |
| Relative size | Smallest | Medium | Largest |
| Typical capacity | Tens of KB per core | Hundreds of KB to a few MB per core | Several MB to tens of MB |
| Access latency | Lowest | Low | Higher |
| Bandwidth | Very high | High | High, but generally lower than L1 |
| Distance from CPU | Closest | Close | Usually shared across cores |
| Typical organization | Instruction and data caches | Often unified | Usually shared |
| Cost per bit | Highest | High | Lower than L1/L2 |
| Main purpose | Immediate CPU data/instructions | Reduce L1 misses | Reduce accesses to DRAM |

Exact capacity, latency and bandwidth vary significantly by processor architecture.

1.3 L1 Cache

L1 is the first cache checked by the processor. Because it is physically close to the execution units and is relatively small, it can provide extremely low access latency.

Many processors divide L1 into:

- L1 Instruction Cache (L1-I): stores recently used instructions.

- L1 Data Cache (L1-D): stores recently accessed data.

A high L1 hit rate is important because instructions and operands can be supplied to the processor without waiting for lower memory levels.

1.4 L2 Cache

L2 is larger than L1 but normally has higher latency. It acts as the next level when the requested information is not found in L1.

An L2 cache reduces the number of accesses to L3 and main memory. Although it is slower than L1, its larger capacity allows it to store a greater working set.

1.5 L3 Cache

L3 is generally the largest cache level in a processor. In many multicore processors, L3 is shared between multiple cores.

A shared L3 cache provides a common high-speed storage area for frequently accessed data. It also reduces the number of requests reaching DRAM.

However, because L3 is larger and often shared, its access latency is higher than L1 and L2.

1.6 Impact on Processor Performance

Cache performance can be represented using the concept of Average Memory Access Time (AMAT).

A simplified two-level expression is:

AMAT = L1 Hit Time + L1 Miss Rate × L2 Miss Penalty

For a multi-level hierarchy:

AMAT = L1 Hit Time + L1 Miss Rate × (L2 Hit Time + L2 Miss Rate × L3/Memory Penalty)

Therefore, a high cache hit rate greatly reduces the effective memory access time.

Performance Analysis

- L1 provides the fastest access and supports high instruction execution rates.

- L2 provides additional capacity while maintaining relatively low latency.

- L3 prevents many requests from reaching DRAM.

- A high L1/L2/L3 hit rate improves processor throughput.

- Cache misses introduce stalls because the processor may have to wait for lower levels of memory.

1.7 Conclusion

L1, L2 and L3 caches work together as a hierarchical memory system. L1 prioritizes extremely low latency, L2 provides a balance between capacity and speed, and L3 provides large shared capacity while still being much faster than main memory. Efficient cache hierarchy design is therefore essential for maintaining processor performance.

2. COMPARISON OF SRAM AND DRAM

2.1 Introduction

SRAM and DRAM are two important types of volatile semiconductor memory. SRAM is commonly used to implement processor caches, whereas DRAM is primarily used as main memory.

The fundamental difference is their storage mechanism. SRAM stores each bit using a circuit containing multiple transistors, while DRAM stores each bit as electrical charge associated with a capacitor.

2.2 Comparison Table

| Parameter | SRAM | DRAM |
| --- | --- | --- |
| Full form | Static Random Access Memory | Dynamic Random Access Memory |
| Storage mechanism | Flip-flop/latch-based cell | Capacitor + transistor |
| Access latency | Very low | Higher |
| Bandwidth | Very high | High |
| Throughput | Very high for low-latency access | High for sequential/burst transfers |
| Refresh required | No | Yes |
| Density | Lower | Higher |
| Cost per bit | Higher | Lower |
| Power characteristics | Higher area/cell cost | Lower cell area but refresh power |
| Typical application | CPU cache | Main memory |

2.3 SRAM

SRAM stores information using a stable circuit state. It does not require periodic refreshing while power is available.

The main advantage of SRAM is its low access latency. This makes it suitable for L1, L2 and other high-speed cache structures.

However, SRAM cells require more transistors than DRAM cells. As a result, SRAM occupies more silicon area and is more expensive per bit.

2.4 DRAM

DRAM uses a capacitor to store charge representing a binary value. Because the stored charge gradually leaks away, the memory must periodically refresh its contents.

DRAM provides significantly higher storage density than SRAM and is therefore suitable for large main-memory capacities.

Although DRAM has greater latency than SRAM, techniques such as burst transfers, row buffering and memory-channel parallelism allow it to provide substantial bandwidth.

2.5 Latency, Bandwidth and Throughput

Latency

SRAM generally provides lower access latency because its storage cells can be accessed without the refresh operations required by DRAM.

DRAM access involves operations such as row activation, column selection and sensing, which contribute to its larger latency.

Bandwidth

DRAM can provide high aggregate bandwidth because modern systems use multiple memory channels and high-speed interfaces.

SRAM has extremely fast access and high internal bandwidth but is limited by its much greater silicon area and therefore smaller practical capacity.

Throughput

SRAM is optimized for rapid individual accesses and is especially valuable when the processor repeatedly accesses small amounts of frequently used data.

DRAM is optimized for providing large amounts of data at relatively high aggregate bandwidth, making it appropriate for the processor's main memory.

2.6 Use in Memory Hierarchy

A typical hierarchy is:

CPU → SRAM Cache → DRAM Main Memory → Storage

SRAM provides speed near the processor, while DRAM provides much greater capacity at lower cost.

2.7 Conclusion

SRAM is faster but expensive and less dense, making it suitable for cache memory. DRAM is slower but much denser and cheaper, making it suitable for main memory. Their complementary characteristics allow computer systems to achieve both high performance and large memory capacity.

3. VIRTUAL MEMORY WITH PAGING VS CACHE MEMORY

3.1 Introduction

Virtual memory and cache memory are both memory-management mechanisms, but they solve different problems.

Cache memory attempts to reduce the average time required to access frequently used data. Virtual memory provides each process with an abstraction of a large address space and allows portions of that address space to be stored outside physical RAM.

Paging is one of the main techniques used to implement virtual memory.

3.2 Comparison Table

| Parameter | Cache Memory | Virtual Memory with Paging |
| --- | --- | --- |
| Main purpose | Reduce memory access latency | Extend address space and provide isolation |
| Managed by | Hardware/cache controller | OS + MMU + hardware |
| Unit | Cache line/block | Page |
| Fast storage | SRAM cache | DRAM |
| Backing storage | Lower cache level/main memory | SSD/HDD or other secondary storage |
| Normal hit latency | Very low | Page-table/TLB overhead even on normal access |
| Miss/fault penalty | Relatively small compared with page fault | Very large for page fault |
| Main benefit | Performance | Capacity, protection and isolation |
| Effect on throughput | Improves CPU throughput | Enables larger workloads but page faults can severely reduce throughput |

3.3 Cache Memory Mechanism

When the processor requests a memory location, the cache checks whether the corresponding cache line is already available.

If the data is present, a cache hit occurs.

If it is absent, a cache miss occurs and the system obtains the data from a lower memory level.

Because cache accesses are extremely fast, a high hit rate significantly improves processor performance.

3.4 Virtual Memory and Paging

Virtual memory divides the virtual address space into fixed-size pages. Physical memory is divided into corresponding page frames.

A page table maps virtual pages to physical frames.

The processor's Memory Management Unit (MMU) translates virtual addresses into physical addresses.

A Translation Lookaside Buffer (TLB) is commonly used to cache recently used address translations.

If the required page is not present in physical memory, a page fault occurs. The operating system must obtain the page from secondary storage and place it into memory.

3.5 Latency Comparison

Cache memory has very low access latency when a hit occurs.

Virtual-memory translation normally adds a small translation overhead, often reduced using the TLB. However, a page fault is extremely expensive because it can require secondary-storage access.

Therefore:

Cache hit → very low latency

TLB hit + memory access → moderate latency

Page fault → extremely high latency

3.6 Effect on System Throughput

Cache memory increases CPU throughput by reducing the time processors spend waiting for data.

Virtual memory allows programs larger than available physical memory to execute and provides process isolation. However, excessive page faults can cause thrashing, where the system spends a large amount of time moving pages rather than executing useful instructions.

3.7 Conclusion

Cache memory primarily improves speed, while virtual memory primarily provides address-space abstraction, protection and efficient use of physical memory. Both are essential but operate at different levels of the memory hierarchy.

4. NAND FLASH VS DRAM

4.1 Introduction

NAND Flash and DRAM are semiconductor memory technologies with significantly different characteristics.

DRAM is volatile and optimized for high-speed random and burst memory access. NAND Flash is non-volatile and is widely used in SSDs, memory cards and embedded storage.

4.2 Comparison Table

| Parameter | DRAM | NAND Flash |
| --- | --- | --- |
| Volatility | Volatile | Non-volatile |
| Typical role | Main memory | Secondary storage |
| Read latency | Low | Higher |
| Write latency | Low | Much higher |
| Random access | Excellent | Less efficient |
| Sequential bandwidth | High | High, especially in modern SSDs |
| Endurance | Very high for normal memory use | Limited program/erase endurance |
| Refresh | Required | Not required |
| Density | High | Very high |
| Cost per bit | Higher | Lower |
| Persistence | Lost when power is removed | Retains data |

4.3 DRAM Characteristics

DRAM provides fast access and high bandwidth. It is directly accessible by the memory controller and is used as the primary working memory of most computer systems.

Its major disadvantages are volatility and the need for refresh operations.

4.4 NAND Flash Characteristics

NAND Flash stores data even when power is removed.

It provides very high storage density and relatively low cost per bit. Modern SSDs combine NAND Flash with controllers, parallel channels and internal caching to achieve high sequential throughput.

However, Flash memory has higher access latency than DRAM, especially for writes and random operations.

Flash also has limited program/erase endurance, so SSD controllers use techniques such as wear leveling and garbage collection.

4.5 Suitability in Hierarchical Memory

A simplified hierarchy is:

CPU Registers → SRAM Cache → DRAM → NAND Flash → Long-Term Storage

DRAM sits closer to the processor because it offers lower latency.

NAND Flash sits lower in the hierarchy because its latency is higher, but it provides much greater capacity and persistence.

4.6 Bandwidth and Throughput

DRAM is designed for rapid repeated memory access and provides high memory bandwidth.

NAND Flash can achieve very high aggregate throughput when several Flash chips and channels operate in parallel. However, its access latency and write behavior remain different from DRAM.

Therefore, NAND Flash is better suited to persistent storage, while DRAM is better suited to active program execution.

4.7 Conclusion

DRAM offers much lower access latency and is suitable for main memory. NAND Flash offers persistence, high density and lower cost per bit, making it suitable for secondary storage. Together, they form important levels of the modern memory hierarchy.

5. COMPARISON OF MRAM AND RRAM

5.1 Introduction

MRAM and RRAM are emerging non-volatile memory technologies designed to provide alternatives to conventional memory technologies.

MRAM stands for Magnetic Random Access Memory and stores information using magnetic states.

RRAM stands for Resistive Random Access Memory and stores information by changing the resistance of a memory cell.

Both technologies are attractive because they can retain data without continuous power.

5.2 Comparison Table

| Parameter | MRAM | RRAM |
| --- | --- | --- |
| Full form | Magnetic Random Access Memory | Resistive Random Access Memory |
| Storage principle | Magnetic state | Resistance state |
| Non-volatile | Yes | Yes |
| Access time | Very low | Potentially very low |
| Read speed | High | High |
| Write speed | High | High, technology-dependent |
| Throughput | High | Potentially high |
| Standby power | Very low | Very low |
| Endurance | Generally high | Technology-dependent |
| Density potential | High | Very high potential |
| Scalability | Good | Very high potential |
| Main challenge | Write energy and density/cost | Variability, endurance and reliability |
| Possible applications | Embedded memory, caches, persistent memory | Embedded memory, neuromorphic and high-density memory |

5.3 MRAM

MRAM stores information using magnetic states rather than electrical charge.

One major advantage is non-volatility. Data can remain stored even when power is removed.

MRAM can offer fast access and high endurance, making it attractive for embedded applications and potentially for memory systems that require persistence.

Its challenges include write energy, cell size, fabrication complexity and achieving high density at competitive cost.

5.4 RRAM

RRAM stores information by changing the resistance of a material.

A cell can be programmed into different resistance states, allowing binary or potentially multi-level storage.

RRAM is attractive because of its potentially small cell size and high density.

It is also being researched for applications such as neuromorphic computing because its resistance can represent different computational states.

However, device-to-device variation, endurance, switching reliability and precise control of resistance states remain important engineering challenges.

5.5 Access Time

Both MRAM and RRAM can provide significantly faster access than conventional non-volatile storage technologies.

MRAM has demonstrated fast read/write behavior and is particularly attractive where low latency and high endurance are required.

RRAM has the potential for very fast switching, although actual performance depends strongly on the particular material, device structure and memory architecture.

5.6 Throughput

MRAM can provide high throughput for embedded memory applications.

RRAM can potentially achieve high density and parallel operation, making it attractive for future high-throughput architectures.

5.7 Overall Comparison

For applications requiring high endurance, fast access and non-volatility, MRAM is an attractive option.

For applications prioritizing very high density, scalability and emerging computing architectures, RRAM has significant potential.

Neither technology universally replaces SRAM or DRAM. Instead, both represent possible additions to future memory hierarchies.