# SIMATS ENGINEERING

Department of Computer Science & Engineering

## ASSESSMENT TOOL 1- CONSTRAINT-BASED PROBLEM SOLVING

| Course Code: | CSA12 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL1- Constraint-Based Problem Solving |
| Weightage: | 40% | Total Marks: | 25 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: | SHIVATHMIKA V | Reg.No.: | 192521028 |
| Department: | B. Tech IT | Date: |  |

### ANNEXURE A

#### Constraint-Based Problem Solving

QUESTION 1: Construct an I/O communication mechanism for a real-time embedded system with the following constraints:

  - CPU utilization must be below 15%

  - Multiple sensors generate interrupts every 1 ms

  - Use vectored interrupt handling

  - Select between memory-mapped and I/O-mapped I/O for optimal latency

The system must keep CPU utilization below 15%, handle multiple sensor interrupts every 1 ms, use

vectored interrupt handling, and select the appropriate I/O technique for minimum latency.

1. Problem Understanding

A real-time embedded system receives data continuously from multiple sensors. Since sensors generate

interrupts every 1 ms, the system must respond quickly without consuming too much CPU time.

- Constraint Analysis

| Constraint | Design Decision |
| --- | --- |
| CPU utilization < 15% | Use efficient interrupt handling and avoid unnecessary polling |
| Multiple sensors every 1 ms | Use interrupt-driven I/O |
| Vectored interrupts required | Assign a separate interrupt vector to each sensor |
| Optimal latency | Use memory-mapped I/O |

3. Proposed Architecture

┌─────────────────┐

│   Sensor 1      │

└────────┬────────┘

│ Interrupt

┌────────▼────────┐

│   Sensor 2      │

└────────┬────────┘

│ Interrupt

┌────────▼────────┐

│   Sensor 3      │

└────────┬────────┘

│

▼

┌──────────────────────┐

│ Interrupt Controller │

│   Vectored Handling  │

└──────────┬───────────┘

│

▼

┌───────────┐

│    CPU    │

└─────┬─────┘

│

Memory-Mapped I/O

│

▼

┌───────────┐

│   Memory  │

└───────────┘

3.Working

- Each sensor is connected to an interrupt line.

- When a sensor produces new data, it generates an interrupt.

- The interrupt controller identifies the source of the interrupt.

- Using vectored interrupt handling, the CPU directly jumps to the corresponding Interrupt Service Routine (ISR).

- The ISR reads the sensor's register using memory-mapped I/O.

- The sensor data is stored in memory for further processing.

- After completing the ISR, the CPU returns to the interrupted task.

- Memory-Mapped I/O vs I/O-Mapped I/O

| Feature | Memory-Mapped I/O | I/O-Mapped I/O |
| --- | --- | --- |
| Address space | Shares memory address space | Separate I/O address space |
| CPU instructions | Normal memory instructions | Special I/O instructions |
| Access speed | Generally efficient | Depends on processor |
| Programming | Simple | More specialized |
| Choice | Selected | Not selected |

Memory-mapped I/O is selected because the sensor registers can be accessed directly using normal

load/store operations, providing a simple and low-latency mechanism.

6. CPU Utilization

To maintain CPU utilization below 15%:

- Use interrupts instead of continuous polling.

- Keep ISRs short.

- Process only the required sensor data inside the ISR.

- Use buffering to handle frequent sensor inputs.

- Give higher priority to time-critical sensors.

7. Justification

The proposed mechanism satisfies all the given constraints. Vectored interrupts provide fast

identification of the interrupt source, while memory-mapped I/O provides efficient access to sensor

registers. Short ISRs and interrupt-driven operation reduce unnecessary CPU activity, helping maintain

CPU utilization below 15% even when sensors generate interrupts every 1 ms

QUESTION 2.  Design an I/O Subsystem

Problem Understanding

The system requires high-speed storage with throughput greater than 3 GB/s. It must use NVMe over

PCIe, while the CPU should not perform bulk data transfer. A DMA controller with interrupt notification is required.

Constraint Analysis:

| Constraint | Design Decision |
| --- | --- |
| Throughput > 3 GB/s | Use high-speed PCIe interface |
| NVMe over PCIe | Use NVMe SSD connected through PCIe |
| CPU should not handle bulk transfer | Use DMA |
| Transfer completion notification | Use DMA interrupt |

Architecture

┌─────────────┐

│     CPU     │

└──────┬──────┘

│ Commands

▼

┌─────────────┐

│ NVMe Driver │

└──────┬──────┘

│

▼

┌─────────────┐

│ PCIe Bus    │

└──────┬──────┘

│

┌──────▼──────┐

│ NVMe SSD    │

└──────┬──────┘

│

▼

┌─────────────┐

│ DMA Engine  │

└──────┬──────┘

│

▼

┌─────────────┐

│ Main Memory │

└─────────────┘

▲

│

DMA Completion

Interrupt

Working

- CPU sends an I/O command to the NVMe controller.

- NVMe SSD communicates through the PCIe interface.

- The DMA controller transfers data directly between the NVMe device and main memory.

- CPU is not involved in transferring every data block.

- After completing the transfer, DMA generates an interrupt.

- CPU handles the interrupt and continues processing.

Justification

NVMe over PCIe with DMA is suitable because it provides high-speed storage communication while

reducing CPU involvement. DMA handles bulk data movement, and interrupt notification informs the

CPU when the operation is completed.

QUESTION 3. Multi-Device Communication Architecture

The system contains 6 USB peripherals and 2 high-speed Thunderbolt devices. It must avoid interrupt starvation, support hardware and software interrupts, and provide fair bus arbitration.

Architecture

┌─────────────┐

│     CPU     │

└──────┬──────┘

│

┌──────────▼──────────┐

│ Interrupt Controller│

└──────────┬──────────┘

│

┌──────────────┴──────────────┐

│                             │

┌──────▼──────┐              ┌───────▼───────┐

│ USB Hub     │              │  Thunderbolt   │

└──────┬──────┘              │    Controller  │

│                     └───────┬────────┘

┌─────┼─────┐                       │

│     │     │                 ┌─────┴─────┐

USB   USB   USB              TB Device  TB Device

1     2     3

│     │     │

USB   USB   USB

4     5     6

Constraint Analysis

- 6 USB devices: Connect through a USB hub/controller.

- 2 Thunderbolt devices: Connect through a dedicated Thunderbolt controller.

- Avoid interrupt starvation: Use interrupt priorities and fair scheduling.

- Hardware interrupts: Generated by physical devices.

- Software interrupts: Used by software for service/background operations.

- Fair bus arbitration: Use round-robin or priority-based arbitration with fairness.

Working

- USB devices communicate through the USB controller/hub.

- Thunderbolt devices use the Thunderbolt controller.

- When devices require CPU attention, interrupts are generated.

- The interrupt controller prioritizes urgent requests.

- A fair arbitration mechanism ensures that lower-priority devices are not permanently blocked.

- Software interrupts can handle background operations.

Justification

The architecture supports all 8 peripherals while preventing interrupt starvation. Fair bus arbitration

ensures that devices receive communication opportunities without allowing one device to monopolize

the bus.

QUESTION4. Data Acquisition System

The system must continuously handle a 500 MB/s data stream, require minimal CPU intervention,

compare DMA with interrupt-driven I/O, and use memory-mapped I/O for device registers.

Architecture

┌─────────────────┐

│ Data Acquisition│

│     Device      │

└────────┬────────┘

│ 500 MB/s

▼

┌─────────────────┐

│ Device Registers│

│ Memory-Mapped   │

│      I/O        │

└────────┬────────┘

│

▼

┌─────────────────┐

│  DMA Controller │

└────────┬────────┘

│

│ Direct Transfer

▼

┌─────────────────┐

│   Main Memory   │

└────────┬────────┘

│

▼

CPU

(Processing only)

DMA vs Interrupt-Driven I/O

| Feature | DMA | Interrupt-Driven I/O |
| --- | --- | --- |
| CPU involvement | Very low | Higher |
| Large data transfer | Excellent | Less efficient |
| 500 MB/s stream | Suitable | Not preferred |
| Data movement | Device ↔ Memory | CPU manages transfer |
| CPU overhead | Low | Higher |

Working

- The data acquisition device continuously generates data at 500 MB/s.

- Its registers are accessed using memory-mapped I/O.

- The DMA controller transfers incoming data directly to main memory.

- CPU does not handle every individual data transfer.

- DMA can notify the CPU after a block transfer is completed.

- CPU processes the stored data.

Justification

DMA is preferred because the system has a continuous high-speed data stream of 500 MB/s. Interrupt

driven I/O would require frequent CPU involvement, whereas DMA transfers large amounts of data

directly to memory and minimizes CPU intervention.

QUESTION 5. Interrupt Handling Mechanism

The system must support nested interrupts, maintain maximum interrupt latency below 10 µs, use

vectored interrupts for high-priority devices, and use software interrupts for background tasks.

Architecture

┌─────────────┐

│     CPU     │

└──────┬──────┘

│

┌───────▼────────┐

│    Interrupt   │

│    Controller  │

└───────┬────────┘

│

┌──────────┼──────────┐

│          │          │

High Priority  Medium    Software

Device       Device     Interrupt

│          │          │

▼          ▼          ▼

Vector 1    Vector 2   Background

│          │         Tasks

└──────────┼──────────┘

▼

CPU

Working

- Devices generate interrupts when service is required.

- The interrupt controller determines the priority of each interrupt.

- A high-priority interrupt is immediately directed to its corresponding vector.

- The CPU starts the appropriate ISR.

- If another higher-priority interrupt occurs, the current ISR can be temporarily suspended.

- The higher-priority ISR executes first.

- After completion, the CPU returns to the interrupted ISR.

- Software interrupts are used for background tasks.

Nested Interrupt Example

Normal Program

│

▼

Low-Priority ISR

│

│ Higher-priority interrupt

▼

High-Priority ISR

│

▼

Return to Low-Priority ISR

│

▼

Return to Normal Program

Maintaining < 10 µs Latency

- Use vectored interrupts for direct ISR access.

- Assign high priority to time-critical devices.

- Keep ISRs short and efficient.

- Allow higher-priority interrupts to preempt lower-priority ISRs.

- Use software interrupts only for background operations.

- Avoid unnecessary processing inside interrupt handlers.

Justification

This mechanism satisfies the requirement for nested interrupts and less than 10 µs maximum interrupt

latency by prioritizing time-critical devices and using vectored interrupt handling. Software interrupts

are reserved for background tasks, preventing them from delaying high-priority hardware interrupts.